Assignment-1 Solution

Name : Smit Makwana

Roll No : 241021

Task A – Voltage Divider Measurement Module

Using two resistors in series, it produces an output voltage that is a predictable 
fraction of the input voltage.

Theoretical formula:
Vout = Vin × (R2 / (R1 + R2))

The Arduino reads the divided voltage through its 10-bit ADC.
ADC-to-voltage conversion:
V = ADC × (5.0 / 1023)

For (10kΩ-10kΩ)
For (4.7kΩ-10kΩ)

For (1kΩ-15kΩ)

Voltage Divider – Theoretical vs Measured Table

| R1 (kΩ) | R2 (kΩ) | Theoretical Vout (V) | Measured Vout (V) | Error (%) |
| --- | --- | --- | --- | --- |
| 10 | 10 | 2.5 | 2.498 | 0.08 |
| 4.7 | 10 | 3.401 | 3.402 | 0.029 |
| 1 | 15 | 4.687 | 4.687 | 0 |

Arduino Code Excerpt for Voltage Measurement

void setup() {

Serial.begin(9600);

}

void loop() {

int adcValue = analogRead(A0);

float voltage = adcValue * (5.0 / 1023.0);

Serial.print("Voltage: ");

Serial.println(voltage, 3);

delay(500);

}

Task B – Capacitance Measurement Using RC Time Constant

When a capacitor charges through a resistor, the voltage across it rises exponentially.
At exactly one time constant (τ = R × C), the capacitor reaches 63.2% of the supply voltage.
This property allows us to measure capacitance by timing the charge interval.
Capacitance calculation:
C = t / R
The Arduino uses analogRead() to monitor voltage and micros() to measure time.
For C = 1 µF

For C = 10 µF

Capacitance Measurement Table

| Capacitor (µF) | Expected Time (ms) | Measured Time (ms) | Calculated C (µF) | Error (%) |
| --- | --- | --- | --- | --- |
| 1 | 10 | 9.932 | 0.993 | 0.7 |
| 10 | 100 | 99.216 | 9.922 | 0.78 |

Arduino Code Excerpt for Capacitance Measurement

const int CHARGE_DISCHARGE_PIN = 8;

const int MEASURE_PIN = A0;

const float KNOWN_RESISTOR = 10000.0;

const float TIME_CONSTANT = 0.632;

const int THRESHOLD_ADC = (int)(5 * TIME_CONSTANT / 5 * 1023.0);

void setup() {

Serial.begin(9600);

}

void loop() {

pinMode(CHARGE_DISCHARGE_PIN, OUTPUT);

digitalWrite(CHARGE_DISCHARGE_PIN, LOW);

delay(100);

pinMode(CHARGE_DISCHARGE_PIN, INPUT);

unsigned long startTime = micros();

int adcValue = 0;

while (adcValue < THRESHOLD_ADC) {

adcValue = analogRead(MEASURE_PIN);

}

unsigned long elapsed_time_us = micros() - startTime;

float time_s = (float)elapsed_time_us / 1000000.0;

float capacitance_F = time_s / KNOWN_RESISTOR;

float capacitance_uF = capacitance_F * 1000000.0;

Serial.print("Time (us): ");

Serial.print(elapsed_time_us);

Serial.print("   Calculated C (uF): ");

Serial.println(capacitance_uF, 3);

delay(3000);

}

Task C – Beginner Ohmmeter Prototype

An ohmmeter determines an unknown resistor by forming a voltage divider with a known 
reference resistor. By measuring the midpoint voltage, the unknown resistance can be computed.

Formula:
R_unknown = R_ref × (Vin / Vout - 1)

For R unknown = 1

For R unknown = 5For R unknown = 10

Ohmmeter – Actual vs Measured Values

| Actual R (Ω) | Measured R (Ω) | Error (%) |
| --- | --- | --- |
| 1000 | 1009.4 | 0.94 |
| 5000 | 5019.6 | 0.392 |
| 10000 | 10014.7 | 0.147 |

Arduino Code Ohmmeter

const int MEASURE_PIN = A0;

const float V_IN = 5.0;

const float R_REF = 5000.0;

void setup() {

Serial.begin(9600);

}

void loop() {

int adc_reading = analogRead(MEASURE_PIN);

if (adc_reading == 0) {

Serial.println("Error: R is extremely large or disconnected.");

delay(1000);

return;

}

float V_out = (float)adc_reading * (V_IN / 1024);

float R_unk_calculated = R_REF * ((V_IN / V_out) - 1.0);

Serial.print("Calculated R: ");

Serial.print(R_unk_calculated, 1);

Serial.println(" Ohms");

delay(2000);

}

Discussion of Errors & Engineering Insights.

The Arduino’s 10-bit ADC divides 0–5 V into 1024 steps (approx 4.88 mV per step).
This causes small jumps in measured voltage, especially in mid-range values and cannot detect changings smaller than 4.88 mV.

Real resistors (±5%) and capacitors (±10–20%) rarely match their printed values.
This leads to mismatch between theoretical and measured outputs even in TinkerCAD.

Multimeters use multiple ranges and attenuators but here my circuit uses only one range, leading to less accuracy and poor performance at extremes.

6. Conclusion

This Assignment help us to apply Ohm’s Law and voltage divider principles and allows us to implement it to measurement circuits. It also deepens the understanding and use of the Arduino ADC to convert analog voltages into digital value by implementing in the problem statement itself. It allows us to enhance understanding the concept of RC time constant and how to measure it effectively and correctly. Overall, this assignment strengthened my foundation not only in Arduino-based measurement techniques but also in the mindset required to build reliable electronic systems.